WEATHERCOAT SMOOTH

Product Description

A premium quality, exterior durable, smooth coating based on a tough, pure and 100% acrylic resin, pigmented with titanium dioxide and other high grade light grade light-fast alkali resisting pigments. Its special properties incorporating fungicides and algaecides ensure outstanding weather resistance and durability. Ideal as a decorative and protective coating for concrete.

Recommended Areas of Application

Area of Use: Exterior

Substrate: Concrete, plaster, masonry, brickwork, blockwork, gypsum, timber.

Technical Data

Sheen Variants: Matt, Smooth

Colours: As per shade card

Volume Solids: 35 ± 2%

| Theoretical Covering Capacity |  |  |
| --- | --- | --- |
| Recommended Dry Film Thickness per coat |  | Theoretical Spread rate |
| Minimum | 45 microns | 7.75 m²/Ltr |
| Maximum | 60 microns | 5.83 m²/Ltr |
| Typical | 50 microns | 7.00 m²/Ltr |

Practical Covering Capacity

For estimation of practical covering capacity, following factors to be taken into account: surface profile, uneven application, over spray, losses in container and equipment etc.

| Drying time | Substrate Temp. | Touch Dry | Overcoating Interval |  |
| --- | --- | --- | --- | --- |
|  |  |  | Minimum | Maximum |
|  | 25°C | 6 hours | 12 hours | Indefinite |
|  | 35°C | 3 hours | 8 hours | Indefinite |
|  | 45°C | 1 ½ Hours | 4 hours | Indefinite |

Application Data

Method of application		:	Brush, roller or conventional spray

Thinner				:	Sweet water

Thinning ratio (by vol.)		:	Use 5% to 10% sweet water depending on the method of application.

Surface Preparation

The performance of this coating is directly related to the degree of surface preparation.

• Ensure surface is clean, dry and free from all contaminants.

• Surface must be sound and free from laitance.

• Remove laitance by wire brush or sweep blast.

Note: Fungal growth should be treated with Bison Fungicidal Solution. After drying, one coat of Plastaseal Penetrating

sealer can be applied if required.

Application Details

• Ensure adequate ventilation during application and drying.

• Do not apply when humidity exceeds 85% and condensation is likely.

• Surface temperature should be 3°C or more above dew point.

• Stir the contents thoroughly before and during use, with a broad flat stirrer, using an upward lifting action.

Brush or Roller : Apply an even coat taking care to avoid runs and sags. Minimum two coats recommended.

TYPICAL PAINT SYSTEM

1 coat x Supreme AR Primer

As required: Tartaruga Fine

2 coats x Weathercoat Smooth

For alternate system suggestion please contact Berger Paints Sales Representative

Storage, Safety Information and Precautions

• Keep out of reach of children.

• Store in cool, dry conditions; ensure adequate ventilation during application.

• Avoid contact with skin and eyes; wear suitable protective clothing such as overalls, goggles, dust masks and gloves.

• Shelf life up to 12 month is from the date of manufacture.

• Flash Point: NA

First-Aid

• Eyes: In the event of accidental splashes, flush eyes immediately with warm water and obtain medical advice.

• Skin: Wash skin thoroughly with soap and water, or with an approved industrial cleaner. Do not use solvents.

• Inhalation: Move patient to fresh air, loosen collar and keep patient rested.

• Ingestion: Do not induce vomiting. Obtain immediate medical attention.

• Dispose in landfill only.

Disclaimer

The information provided in this data sheet is not intended to be complete. It is the responsibility of the user to ensure that the product used is suitable for the purpose it is meant to serve. In view of various factors affecting the performance of the coating, we cannot assume responsibility for the product performance, nor do we accept any liability arising from use of this product, unless specifically agreed to in writing by us. The information provided in the datasheet may be modified by us without notice.