SYSTEM : ECONOMY EMULSION SYSTEM FOR INTERIORS

Concrete and Masonry Surface - Interior

System Description

Primary Coat: 1 Coat of Berger Value PVA Primer

A water based primer based on PVA co-polymer, formulated to penetrate into and seal the surface.

Volume Solids: 18 ± 2%

VOC: <0.01(g/L) (Test Method -GC FID)

Secondary Coat: 2 Coats of Berger Value Stucco

An acrylic co-polymer based filler containing alkali resisting pigments, for the filling of surface irregularities and indentations, hairline surface cracks etc.

Volume Solids: 52% ± 2%

VOC: Near 0 (Test Method - ISO 11890 - 1)

Estidama QCC approved

Conform to MPI 4

Top Coat: 2 Coats of Berger Select Acrylic Emulsion

Emulsion with good washability and scrub resistance and offers Smooth and Matt finish

Volume Solids: 34 % ± 2 %

Heavy Metals Free

Areas of Application

Area of Use: Interior

Substrate: Concrete, Plaster, Masonry, Gypsum, Timber