Outdoor Multisport Scoreboard BT2025 P Beach Soccer

The outdoor digital scoreboard shall display the game clock, each team score and the period number.

The scoreboard shall be able to manage, as a minimum, the following sports: beach soccer, football, rugby, beach volley, field hockey.

- LED technology

- High luminosity LED technology shall be used to ensure proper readability in all outdoor light conditions and provide a viewing angle of 160°.

- The luminosity level of the red LED display shall be adjustable from the controlling keypad at any time during the game.

- Game clock and scores

- The game clock and the scores shall be displayed with 25cm digits and shall be readable from as far as 120m.

- A four-digit game clock shall display minutes and seconds, be capable of counting up or down and, when the game is not in progress, be able to display the local time and optionally the temperature.

- The teams’ score shall be arranged on 1 line.

- Two digits shall be use to display the teams’ score from 0 to 99.

- One 20cm digit shall display the period number and shall be located between the scores.

- Teams’ names

- The HOME and GUEST names shall be arranged on 1 line and shall be made of adhesive letters stuck to the front cover.

- Technical characteristics

- The scoreboard shall have a weatherproof aluminium casing rated no less than IP54 for outdoor use and equipped with a transparent shockproof protective front cover providing an IK07 rating.

- Fan easy access to all components the front cover shall be removable by sliding out of either side providing.

- Tropicalized electronics shall be required in order to guarantee reliable outdoor operation in temperatures ranging from -20 to +50°C and in high humidity conditions.

- To power the scoreboard a mains of 110 to 230V, 50/60Hz. shall be required.

- The scoreboard shall weigh no more than 34kg while measuring approximately 1500mm x 1150mm x 85mm.

- A multisport digital handheld keypad shall be used to control the scoreboard a radio signal.

The power capability of the 230V power source provided by the installer shall be no less than 35W.

Required accessory

- Handheld digital keypad

- Technical characteristics

- The keypad shall control the scoreboard via 868MHz HF radio signal.

- Two AA (1.5V) batteries shall be required to power the keypad.

- The keypad shall be able to be used indoor or outdoor.

- Keypad functions

The keypad shall perform the following operations:

- Sport selection and configuration

- Start a new match

- Reset the game clock

- Start and Stop the game clock

- Increment and decrement the Home and Guest scores

- Select the period number

- Selection of the game clock counting mode: up or down

- Selection of game clock display or local time display

- Adjustment of the scoreboards luminosity

- Time setting

- Turn ON and OFF an optional horn.

Options

When post mounting is required, posts sized and designed to face the local wind conditions shall be available.

A solar power supply shall be available as an option to power the scoreboard.